Registration form

DISTRIBUTION 8 HOURS    

48 HOUR RUSH ORDER PROCESSING FEE ADDITIONAL $50.00

Start and Finish Dates: _______________________________________________________
You will have 90 days from this date in order to complete this course




List number of hours worked on assignment must match State Requirement.  ____________
Name________________________________Signature_______________________________

I have read and understood the disclaimer notice on page 2.  Digitally sign XXX

Address____________________________________________________________________

City_______________________________State_____________________Zip______________

Email__________________________________ Fax (_____) __________________________

Phone:


Home (_____) ______________________       Work (_____) __________________________

Operator ID #_________________________________Exp. Date____________________

Please circle/check which certification you are applying the course CEU’s.

Water Distribution ___    Water Treatment ___   Other ___________________________

Technical Learning College   TLC PO Box 3060, Chino Valley, AZ 86323
Toll Free (866) 557-1746    Fax (928) 272-0747   info@tlch2o.com 
If you’ve paid on the Internet, please write your Customer#_________________________

Please invoice me, my PO#____________________________________________________

Please pay with your credit card on our website under Bookstore or Buy Now.  Or call us and provide your credit card information.

We will stop mailing the certificate of completion, so we need either your fax number or e-mail address.  We will e-mail the certificate to you, if no e-mail address; we will fax it to you.  
 DISCLAIMER NOTICE AND AFFIDAVIT OF EXAM COMPLETION

Technical Learning College CEU Course Policies

Disclaimer Notice

I understand that it is my responsibility to ensure that this Continuing Education Unit (CEU) course is either approved or accepted in my State for CEU credit. I acknowledge that State laws and rules change frequently, and I believe this course is currently accepted in my State for CEU or contact hour credit. If it is not, I will not hold Technical Learning College responsible. Furthermore, I recognize that this type of study program deals with dangerous conditions, and I will not hold Technical Learning College, Technical Learning Consultants, Inc. (TLC), The Love of Christ Bible College liable for any errors, omissions, or advice contained in this CEU education training course, nor for any violation, injury, neglect, or damage caused by this CEU education training, course material suggestion, or error. If I require assistance, I will contact TLC and double-check that my registration page and assignment have been received and graded.

State Approval Information

Please refer to the State Approval Listing Link to verify if your State accepts or has pre-approved this course. Note that not all States or courses are listed. If the course is not accepted for CEU credit, Technical Learning College will provide the course free of charge if you request your State to accept it for credit.

Professional Engineers: Most states will accept our courses for credit, but we do not officially list the States or Agencies. Please check with your State for approval.

State Approval Listing URL: 

http://www.tlch2o.com/downloads/PDF/CEU%20State%20Approvals.pdf
Course Manual-Printed Version

You may obtain a printed version of the course manual from TLC for an additional $129.95, plus shipping charges.

Affidavit of Exam Completion

I affirm that I personally completed the entire text of the course. I also affirm that I completed the exam without assistance from any outside source. I understand that it is my responsibility to file or maintain my certificate of completion as required by the State or by the designation organization.

Grading Information

To maintain the integrity of our courses, we do not distribute test scores, percentages, or information on questions missed. Our exams are based on pass/fail criteria, with the benchmark for successful completion set at 70%. Once you pass the exam, your record will reflect a successful completion, and a certificate will be issued to you.

Rush Grading Service

If you need your assignment graded and results mailed to you within a 48-hour period, an additional rush service handling fee of $50.00 will apply. Please note that this fee may not cover postage costs. To request this service, write "RUSH" at the top of your Registration Form. We will prioritize your assignment in the grading and processing line.

For security purposes, please fax or e-mail a copy of your driver’s license and always call us to confirm we’ve received your assignment and to confirm your identity. 

CERTIFICATION OF COURSE PROCTOR
Technical Learning College requires that students who takes a correspondence or home study program course must pass a proctored course reading, quiz and final examination.  The proctor must complete and provide the school a certification form for each examination administered by the proctor.

	Instructions.  When a student completes the course work, fill out the blanks in this section and provide the form to the proctor with the examination.


Name of Course:






 

Name of Licensee:






  

	Instructions to Proctor.  After an examination is administered, complete and return this certification and examination to the school in a sealed exam packet or in pdf format.


I certify that:

1.
I am a disinterested third party in the administration of this examination.  I am not related by blood, marriage or any other relationship to the licensee which would influence me from properly administering the examination.  

2.
The licensee showed me positive photo identification prior to completing the examination.

3.
The enclosed examination was administered under my supervision on 

.  The licensee received no assistance and had no access to books, notes or reference material.

4.
I have not permitted the examination to be compromised, copied, or recorded in any way or by any method.

5.  Provide an estimate of the amount of time the student took to complete the assignment. 

Time to complete the entire course and final exam. _____________________________

Notation of any problem or concerns:

Name and Telephone of Proctor (please print):

Signature of Proctor

Distribution 8 Hours CEU Course Answer Key
Name____________________________________
Phone ___________________________________





Did you check with your State agency to ensure this course is accepted for credit?

You are responsible to ensure this course is accepted for credit. No refunds.
Method of Course acceptance confirmation. Please fill this section 

Website __ Telephone Call___ Email____  Spoke to_________________________

Did you receive the approval number, if applicable?  ________________________

What is the course approval number, if applicable?  _________________________

You can electronically complete this assignment in Adobe Acrobat DC.

Please Circle, Bold, Underline or X, one answer per question. A felt tipped pen works best.
1. A B C D 

2. A B C D 

3. A B C D 

4. A B C D 

5. A B C D 

6. A B 

7. A B 

8. A B 

9. A B C D

10. A B C D 

11. A B C D 

12. A B 

13. A B C D 

14. A B C D 

15. A B C D 

16. A B C D 

17. A B 

18. A B C D 

19. A B C D 

20. A B C D 

21. A B C D 

22. A B C D 

23. A B C D 

24. A B C D 

25. A B C D 

26. A B C D 

27. A B 

28. A B C D 

29. A B C D 

30. A B C D 

31. A B 

32. A B C D

33. A B C D 

34. A B C D 

35. A B C D 

36. A B 

37. A B 

38. A B 

39. A B 

40. A B 

41. A B 

42. A B C D 

43. A B C D 

44. A B C D 

45. A B C D 

46. A B C D 

47. A B C D 

48. A B C D 

49. A B C D 

50. A B C D 

51. A B C D 

52. A B 

53. A B 

54. A B C D 

55. A B 

56. A B 

57. A B C D 

58. A B 

59. A B C D 

60. A B C D 

61. A B C D 

62. A B 

63. A B C D 

64. A B 

65. A B C D 

66. A B 

67. A B C D 

68. A B C D 

69. A B 

70. A B C D 

71. A B C D 

72. A B C D 

73. A B C D 

74. A B C D 

75. A B 

76. A B C D 

77. A B C D 

78. A B C D

79. A B C D 

80. A B C D 

81. A B C D 

82. A B C D 

83. A B C D 

84. A B C D 

85. A B C D 

86. A B C D 

87. A B C D 

88. A B C D 

89. A B C D 

90. A B C D 

91. A B C D 

92. A B C D

93. A B C D 

94. A B C D 

95. A B C D 

96. A B C D 

97. A B C D 

98. A B C D 

99. A B C D 

100. A B C D 

101. A B C D

102. A B C D 

103. A B C D 

104. A B C D 

105. A B C D 

106. A B C D 

107. A B C D 

108. A B C D 

109. A B C D 

110. A B C D 

111. A B C D 

112. A B C D 

113. A B C D 

114. A B C D 

115. A B 

116. A B 

117. A B 

118. A B 

119. A B 

120. A B 

121. A B 

122. A B 

123. A B 

124. A B C D

125. A B C D 

126. A B C D 

127. A B C D 

128. A B C D 

129. A B C D 

130. A B C D 

131. A B C D 

132. A B C D 

133. A B C D 

134. A B C D 

135. A B C D 

136. A B 

137. A B C D 

138. A B C D 

139.  A B 

140. A B 

141. A B 

142. A B C D 

143. A B C D 

144. A B C D 

145. A B 

146. A B 

147. A B C D

148. A B C D 

149. A B C D 

150. A B C D 

151. A B C D 

152. A B C D 

153. A B C D 

154. A B C D 

155. A B C D 

156. A B C D 

157. A B C D 

158. A B C D 

159. A B C D 

160. A B 

161. A B C D 

162. A B C D 

163. A B C D 

164. A B C D 

165. A B C D 

166. A B C D 

167. A B 

168. A B 

169. A B C D 

170. A B C D

171. A B C D 

172. A B C D 

173. A B C D 

174. A B C D 

175. A B C D 

176. A B C D 

177. A B 

178. A B C D 

179. A B 

180. A B C D 

181. A B C D 

182. A B 

183. A B 

184. A B C D 

185. A B C D 

186. A B C D 

187. A B C D 

188. A B C D 

189. A B C D 

190. A B C D 

191. A B C D 

192. A B C D 

193. A B C D

194. A B C D 

195. A B C D 

196. A B C D 

197. A B 

198. A B C D 

199. A B C D 

200. A B 

Please e-mail or fax this survey along with your final exam

DISTRIBUTION 8 HOURS CEU COURSE

CUSTOMER SERVICE RESPONSE CARD

NAME: ____________________________________________________________ 

E-MAIL_______________________________PHONE_______________________

PLEASE COMPLETE THIS FORM BY CIRCLING THE NUMBER OF THE APPROPRIATE ANSWER IN THE AREA BELOW.

Please rate the difficulty of your course.

Very Easy  
0
1
2
3
4
5     Very Difficult

Please rate the difficulty of the testing process.

 Very Easy  
0
1
2
3
4
5     Very Difficult

Please rate the subject matter on the exam to your actual field or work.

Very Similar  
0
1
2
3
4
5     Very Different

How did you hear about this Course? ____________________________________________

What would you do to improve the Course?

____________________________________________________________________________ 

Any other concerns or comments. 

Disclaimer Notice
I understand that I am 100 percent responsible for ensuring that TLC receives the Assignment and Registration Key.   I understand that TLC has a zero tolerance towards not following their rules, cheating or hostility towards staff or instructors.  I need to complete the entire assignment for credit.  There is no credit for partial assignment completion.  My exam was proctored. I will contact TLC if I do not hear back from them within 2 days of assignment submission. I will forfeit my purchase costs and will not receive credit or a refund if I do not abide by TLC’s rules.  I will not hold TLC liable for any errors, injury, death or non-compliance with rules.  I will abide with all federal and state rules and rules found on page 2.  

Please Sign that you understand and will abide with TLC’s Rules.

______________________________________________________

Signature

Please write down any questions you were not able to find the answers or that have errors.  

When Finished with Your Assignment

REQUIRED DOCUMENTS

Please scan the Registration Page, Answer Key, Survey and Driver’s License and email it to info@TLCH2O.com.

IPhone Scanning Instructions

If you are unable to scan, take a photo of these documents with your iPhone and send these photos to TLC, info@TLCH2O.com.

FAX

If you are unable to scan and email, please fax these to TLC, if you fax, call to confirm that we received your paperwork. (928) 468-0675

This course contains general EPA’s SDWA federal rule requirements.  Please be aware that each state implements water / sampling procedures/ safety / environmental / SDWA regulations that may be more stringent than EPA’s regulations.  Check with your state environmental/health agency for more information. These rules change frequently and are often difficult to interpret and follow. Be careful to be in compliance with your regulatory agencies and do not follow this course for any compliance concerns. 

Distribution 8 Hours CEU Training Course Assignment

The Distribution 8 hours CEU course assignment is available in Word on the Internet for your convenience, please visit www.ABCTLC.com and download the assignment and e-mail it back to TLC.  

You will have 90 days from receipt of this manual to complete it in order to receive your Professional Development Hours (PDHs) or Continuing Education Unit (CEU).  A score of 70 % or better is necessary to pass this course.  If you should need any assistance, please email or fax all concerns and the completed ANSWER KEY to info@tlch2o.com. 

Select one answer per question.  Please utilize the answer key.  
(s) on the answer will indicate either plural and singular tenses.  

Please write down any questions you were not able to find the answers or that have errors.  

Hyperlink to the Glossary and Appendix

http://www.abctlc.com/downloads/PDF/WTGlossary.pdf

Water Distribution Section



System Elements

1.   In the distribution system, storage reservoirs are structures used to store water and _____________ the supply or pressure. 

A.  Equalize


C.  Provide a reserve pressure for

B.  Increase water pressure
D.  None of the above

Butterfly Valve

2.  Butterfly valves are rotary type of valves usually found on large transmission lines, and may also have an additional valve beside it known as a___________ to prevent water hammer.

A.  Regulator

C. Bypass

B.  PRV

D.   None of the above

Water Distribution Valves

3.  For large shutoff valves, it is necessary to surround the valve operator or entire valve within a vault or manhole to allow?

A.  Bluestakes

C.  Repair or replacement

B.  Testing

D.  None of the above

Gate Valves

4.  If the valve is wide open, the gate inside the valve is _______________ into the valve bonnet. 

A.  Fully drawn up
C.  Fully closed

B.  Fully down

D.  None of the above

5.  There is little pressure drop or flow restriction through gate valves; however, gate valves are not suitable for?

A.  Throttling purposes
C.  Pressure drops
B.  Isolation 


D.  None of the above

Ball Valves

6.  Ball valves should be either fully-on or fully-off, some ball valves also contain a swing check located within the ball to give the valve a check valve feature. 

A.  True
B.  False

Water Pressure

7.  2.31 feet of water is equal to 1 psi, or 1 foot of water is equal to about a half a pound (.433 pounds to be exact).

A.  True
B.  False

8.  For ordinary domestic use, water pressure should be between 25 and 45 psi. 

A.  True
B.  False

9.  20 psi is the minimum pressure required at any point in the water system, so that ____________________ is prevented.

A.  Cavitation

C.  Backflow and infiltration
B.  Back pressure
D.   None of the above
10.  Which of the following is provided from the direct force of the water, or by the height of the water? 

A.  Pressure

C.  Maximum daily use
B.  System integrity
D.   None of the above
Water Use or Demand

11.  Water system demand comes from many sources including residential, commercial, industrial and public consumers as well as waste and some? 

A.  Pressure

C.  Unavoidable loss

B.  System integrity
D.   None of the above

12.  The combination of storage reservoirs and distribution lines must be capable of meeting consumers’ needs for pressure at all times. 

A.  True
B.  False

Water Storage Introduction

13.  Which of the following prevents contamination of water as it travels to the customer, finished water storage facilities are an important component of the protective distribution system?

A.  Cathodic protection
C.  Barrier
B.  Corrosion protection
D.  None of the above  

Storage and Distribution

14.  Proper construction is important in maintaining system integrity and the distribution system must also protect? 

A.  Cathodic protection
C.  Water quality
B.  Corrosion protection
D.  None of the above  

Water Storage Facilities 

15. Water storage facilities and tanks vary in different types that are used in the water distribution systems, such as stand pipes, elevated tanks and reservoirs, hydropneumatic tanks and?

A.  Surge tanks


C.  Storage reservoirs


B.  Water distribution systems 
D.  None of the above  

(S) Means the answer can be plural or singular in nature
Storage Reservoirs

16.  The text recommends that _____________ be located at a high enough elevation to allow the water to flow by gravity to the distribution system. 

A.  Storage reservoirs


C.  Tree systems
B.  Levelers


 
D.  None of the above  

Steel Reservoirs

17.  Steel reservoirs or tanks generally have higher construction and installation costs than concrete, and require less maintenance. 

A.  True
B.  False

Cross-Connection Section

What is Backflow? 

18.  Backflow is the undesirable reversal of flow of nonpotable water or other substances through a ____________________and into the piping of a public water system or consumer’s potable water system. 

A.  Backflow


C.  Cross-connection

B.  Indirect connection
D.  None of the above
19.  Which can occur when there is a stoppage of water supply due to nearby firefighting, a break in a water main? 

A.  Backsiphonage

C.  Cross-connection

B.  Backpressure

D.  None of the above
20.  Which is a type of backflow caused by a downstream pressure that is greater than the upstream or supply pressure in a public water system or consumer’s potable water system?

A.  Backflow

C.  Indirect connection

B.  Backpressure
D.  None of the above
21.  Which can result from an increase in downstream pressure, a reduction in the potable water supply pressure, or a combination of both? 

A.  Backflow

C.  Backsiphonage

B.  Backpressure
D.  None of the above
22.  Which can have two forms-backpressure and backsiphonage?  

A.  Backflow

C.  Cross-connection

B.  Backpressure
D.  None of the above
23. The basic mechanism for preventing backflow is a mechanical _____________, which provides a physical barrier to backflow. 

A.  Air gap


C.  Backflow

B.  Backflow preventer 
D.  None of the above
24. The principal types of mechanical backflow preventer are the reduced-pressure principle assembly, the _________________, and the double check valve assembly. 

A.  Vacuum breaker
C.  Backflow check

B.  Air gaper

D.  None of the above
25. Which of the following is a means or mechanism to prevent backflow? 

A.  Check device or method

C.  Backwater check valve

B.  Backflow preventer 

D.  None of the above
26. According to the text, basic means of preventing backflow is a(n) ______________, which either eliminates a cross-connection or provides a barrier to backflow. 

A.  Vacuum

C.  Backflow check

B.  Air gap

D.  None of the above
Types of Backflow Prevention Methods and Assemblies

27.  The type of device selected for a particular backflow installation depends on several factors.

A.  True
B.  False

28.  When the ________________is restricted, such as the case of an air gap located near a wall, the air gap separation must be increased. 

A.  Air break


C.  Airflow
B.  Barrier to backflow 
D.  None of the above
29.  An air gap is a physical disconnection between the free-flowing discharge end of a potable water pipeline and the top of a(n)? 

A.  Open receiving vessel
C.  Barrier to backflow
B.  Air break


D.  None of the above
30.  Which of the following must be at least two times the diameter of the supply pipe and not less than one inch?

A.  Open receiving vessel
C.  Air gap

B.  Air break


D.  None of the above
31.  An air break is a physical separation between the free-flowing discharge end of a potable water supply pipeline, and the overflow rim of an open or non pressure receiving vessel. 

A.  True
B.  False

32.  According to the text, air gap separations must be vertically orientated a distance of at least twice the inside diameter of the supply, but never less than? 

A.  1 inch 

C.  12 inches
B.  2 inches

D.  None of the above
33.  An obstruction around or near an _________________ may restrict the flow of air into the outlet pipe and nullify the effectiveness of the air gap to prevent backsiphonage. 

A.  Open receiving vessel
C.  Air gap

B.  Air break


D.  None of the above
34.  An air gap is acceptable for _________________ and is theoretically the most effective protection. 

A.  High hazard installations

C.  Low pollutional hazards

B.  High pollutional concerns

D.  None of the above
Vacuum Breakers

35.  Which of the following devices can have two primary types: atmospheric and pressure. 

A.  Vacuum breaker(s)


C.  Hazard application(s)

B.  Atmospheric vacuum breakers

D.  None of the above
36.  Vacuum breakers device’s primary purpose is to protect the water system from cross connections due to submerged inlets, such as irrigation systems and tank applications.  

A.  True
B.  False

37.  If either reduced pressure backflow assembly check valve leaks, the differential pressure relief valve maintains a differential pressure of at least two (2) psi between the supply pressure and the zone between the two check valves by discharging water to atmosphere. 

A.  True
B.  False

38.  According to the text, the reduced pressure backflow assembly or RP is designed to prevent backflow caused by backpressure and backsiphonage from low to high health hazards. 

A.  True
B.  False

39.  According to the text, the RP needs to installed 12 inches above the ground for testing purposes only.

A.  True
B.  False

40.  The reduced pressure backflow assembly can be used for high hazard situations under backpressure only. Under normal conditions, the second check valve should never close. 

A.  True
B.  False

41. According to the text, if the second check valve fails or becomes fouled and backflow into the reduced pressure zone occurs, the relief port vents the backflow to atmosphere.

A.  True
B.  False

42.  According to the text, the reduced pressure zone port opens anytime pressure in the zone comes within 2 psi of the supply pressure.

A.  True
B.  False

Pump and Motor Section  

Common Hydraulic Terms

43.  Which of the following definitions is engineering science pertaining to liquid pressure and flow?

A.  Hydraulics

C.  Hydrokinetics

B.  Hydrology

D.  None of the above

44.  Which of the following definitions is the pressure exported by the atmosphere at any specific location? 

A.  Pressure, Atmospheric
C.  Pressure, Gauge

B.  Pressure, Static

D.  None of the above

45.  Which of the following definitions is pressure above zero absolute, i.e. the sum of atmospheric and gauge pressure?

A.  Pressure, Atmospheric
C.  Pressure, Gauge

B.  Pressure, Static

D.  None of the above

46.  Which of the following definitions is the force per unit area, usually expressed in pounds per square inch?

A.  Pressure, Absolute
C.  Pressure, Gauge

B.  Pressure


D.  None of the above

47.  Which of the following definitions is the pressure differential above or below ambient atmospheric pressure?

A.  Pressure, Absolute
C.  Pressure, Gauge

B.  Pressure


D.  None of the above

48.  Which of the following definitions is height of a column or body of fluid above a given point expressed in linear units? 

A.  Head, Backpressure 
C.  Head 


B.  Head, Suction

D.  None of the above

49.  Which of the following definitions is required to overcome the friction at the interior surface of a conductor and between fluid particles in motion? 

A.  Head, Friction 
C.  Head particles 


B.  Head, Suction 
D.  None of the above

50.  Which of the following definitions is the pressure in a fluid at rest?

A.  Head, Pressure
C.  Head Suction


B.  Pressure, Static
D.  None of the above

51.  Which of the following definitions is the height of a column or body of fluid above a given point?

A.  Head, Friction 
C.  Head suction


B.  Head, Static 
D.  None of the above

52.  Sea level pressure is approximately 2.31 pounds per square inch absolute, 1 bar = .433psi.

A.  True
B.  False
General Pumping Fundamentals

53.  Here are the important points to consider about suction piping when the liquid being pumped is below the level of the pump: Sometimes suction lift is also referred to as ‘positive suction head’.

A.  True
B.  False
54.  According to the text, suction lift is when the level of water to be pumped is below the?

A.  Impeller
 
C.  Centerline of the pump

B.  Suction

D.  None of the above

55.  The suction side of pipe should be one diameter smaller than the pump inlet. 

A.  True
B.  False
56.  The required eccentric reducer should be turned so that the top is flat and the bottom tapered.

A.  True
B.  False
Pumps

57.  Pumps are excellent examples of? 

A.  Hydrostatics 


C.  Multi-stage pumps

B.  Quasi-static devices

D.  None of the above

58. Positive displacement pumps have a piston (or equivalent) moving in a closely-fitting cylinder and forces are exerted on the fluid by motion of the piston. 

A.  True
B.  False

59.  More complicated pumps have valves check valves that open to allow _________, and close automatically to prevent reverse flow. 

A.  Pistons 

C.  Passage in one direction

B.  Diaphragms
D.  None of the above

(S) Means the answer can be plural or singular in nature
Pump Categories

60.  The key to understanding a pump’s operation is that a pump is to move water and generate the ___________ we call pressure. 

A.  Delivery force

C.  Diaphragm pressure

B.  Impeller force

D.  None of the above

61.  With a centrifugal pump the pressure is not referred to in pounds per square inch but rather as the equivalent in elevation, called?

A.  Inward force 

C.  Delivery force


B.  Head


D.  None of the above

62.  According to the text, pumps may be classified based on the application they serve. 

A.  True
B.  False

Basic Water Pump

63.  The centrifugal pumps work by spinning water around in a circle inside a?

A.  Vortex
 
C.  Cylindrical pump housing

B.  Cylinder

D.   None of the above

64.  As the water slows down and its kinetic energy decreases, that water's pressure potential energy increases. 

A.  True
B.  False

65.  In the operation of the pump, the water at the edge of the _____________ inward on the water between the impeller blades and makes it possible for that water to travel in a circle. 

A.  Inward force 

C.  Center of the impeller

B.  Pump pushes

D.  None of the above

Types of Water Pumps

66.  The water production well industry almost exclusively uses Turbine pumps, which are a type of centrifugal pump.  

A.  True
B.  False

67.  The size and number of stages, horsepower of the motor and _____________are the key components relating to the pump’s lifting capacity.  

A.  Pumping head

C.  Horsepower

B.  Atmospheric pressure
D.  None of the above 

68.  The shaft turns the impellers within the pump housing while the?  

A.  Desired pumping rate is obtained

C.  Water moves up the column

B.  Horsepower turns the shaft

D.  None of the above

69.  The rotating shaft in a line shaft turbine is actually housed within the column pipe that delivers the water to the surface.   

A.  True
B.  False

70.  The size of the ________________are selected based on the desired pumping rate and lift requirements.  

A.  Valve

C.  Column, impeller, and bowls

B.  Lantern bearing 
D.  None of the above

Safety Section 
Excavation and Trenching Section

71.  According to the text, the _____________________ was revised because excavating is the most dangerous of all construction operations. 

A.  Competent rule


C.  Emergency rule

B.  OSHA excavation standard
D.  None of the above

72.  OSHA also revised the __________________ to clarify the requirements.

A.  Competent rule

C.  Protective equipment standard

B.  Existing standard

D.  None of the above

73. The performance criteria in the new standard provides employers with options when classifying soil and when selecting methods to protect the _____________ from cave-ins.

A.  Competent person 
C.  Construction equipment

B.  Employee


D.  None of the above

74.  Although employers have options when meeting some of the requirements, __________ must realize that the employee must be protected at all times.

A.  Competent persons 
C.  Contractors

B.  Employers


D.  None of the above

75.
Professional engineers will be required in some situations to plan or design the excavation and/or method of protecting the worker.

A.  True
B.  False

Competent Person

76.
The __________has authorization to take prompt corrective measures to eliminate identified hazards.

A.  Competent person 
C.  Watchman

B.  Contractor


D.  None of the above

77.
A _______________ must have specific training in and be knowledgeable about soils analysis, the use of protective systems and the requirements of 29 CFR Part 1926.650-652 Subpart P.

A.  Competent person 

C.  Watchman

B.  Contractor 



D.  None of the above

78.
Everyone is required to practice _____________________ one a year. 

A.  Competent person training
C.  Emergency procedures

B.  Rescue training exercises

D.  None of the above

Competent Person Duties

79.
The competent person performs daily inspections of the protective equipment, ________________, safety equipment, and adjacent areas.  

A.  Work progress


C.  Trench conditions

B.  Construction Crew


D.  None of the above

80.
The competent person shall make ______________ prior to the start of work and as needed throughout the shift. 

A.  Personnel assignments
C.  Inspections

B.  Training available

D.  None of the above

81.
The competent person shall make ______________ after every rainstorm or other hazard occurrence.

A.  Inspections

C.  Protective equipment available

B.  Training available

D.  None of the above

82.
The competent person must have knowledge of ______________________, telephone or radio dispatch.

A.  Personnel assignments
C.  Emergency contact methods

B.  Work schedules

D.  None of the above

83.
The competent person removes employees and ______________________ from hazardous conditions and makes all changes necessary to ensure their safety.

A.  Competent persons 

C.  Protective equipment

B.  All other personnel 

D.  None of the above

84.
The competent person makes sure that all _____________ have proper protective equipment, hard-hats, reflective vests, steel-toed boots, harnesses, eye protection, hearing protection and drinking water.

A.  Competent persons 
C.  Employees

B.  Contractors

D.  None of the above

Scope of Work

85.
According to the text, during excavation work a competent person shall be on the job site at all times when personnel are working within or around the _____________.

A.  Competent person 
C.  Excavation


B.  Contractors

D.  None of the above

86.
Prior to opening an excavation, the estimated locations of ______________________ that reasonably may be expected to be encountered during excavation work shall be determined.  

A.  Unauthorized persons 
C.  Underground utility installations

B.  Employees


D.  None of the above

87.
__________________ shall be taken to protect employees against the hazards posed by water accumulation in the excavation.

A.  Additional care


C.  Ladders

B.  Adequate precautions

D.  None of the above

88.
In trench excavations that are four (4') feet or more in depth, a stairway, ladder, or ramp shall be used as a ________________.

A.  Tool



C.  Bridge

B.  Means of access or egress
D.  None of the above

89.
When excavations are made in vehicular traffic areas, _______________ shall wear a warning vest made with reflective material or highly visibility material.

A.  Competent persons

C.  Rescue personnel

B.  Each employee


D.  None of the above

90.
The air shall be tested in excavations where __________________ exist, or could be reasonably expected to exist.

A.  Limited visibilities

C.  Oxygen deficiency or gaseous conditions

B.  Employees


D.  None of the above

91.
When the atmosphere contains less than 19.5 percent oxygen, the area must be continuously ventilated until the _____________________.

A.  Excavation is closed

C.  Oxygen levels are above 19.5 percent

B.  Employees enter the space
D.  None of the above

92.
Where a _____________________, the area shall be ventilated until the flammable gas concentration is below 20 percent of the LFL (lower flammable limit).

A.  Competent person requires monitoring
C.  Worker encounters fumes

B.  Gaseous condition exists


D.  None of the above

93.
Whenever ___________________ exist or could reasonably exist, the air must be monitored continuously to assure that workers are protected.

A.  Traffic conditions

C.  Oxygen deficiency or gaseous conditions

B.  Excavations

D.  None of the above

94.
Where the stability of adjoining buildings, walls or other structures are ______________, shoring, bracing, or underpinning shall be provided to ensure the stability of such structures for the protection of employees.

A.  Not a concern



C.  Endangered by excavation operations

B.  Not mentioned in the specifications
D.  None of the above

95.
In situations where sidewalks, pavement and appurtenant structures may be undermined, a support system such as shoring must be provided to protect ____________ from the possible collapse of such structures.

A.  Unauthorized persons
C.  Vehicles

B.  Employees


D.  None of the above

Personnel Protective Systems

96.
According to the text, employees in _______________ shall be protected from cave-ins by an adequate protective system, which shall be inspected by a competent person.

A.  Excavations
C.  Protective systems

B.  Vehicles

D.  None of the above

97.
The use of ________________ is required for all excavations deeper than five (5') feet, except when excavation is within stable rock.

A.  Tables 


C.  Protective systems
B.  Tabulated data

D.  None of the above

98.
For trench excavations less than five (5') feet deep, the use of _________________ may not be required unless there is evidence of a potential cave-in.  The competent person shall make this determination.

A.  Ladders


C.  Ramps  
B.  Protective systems
D.  None of the above

99.
Requirements for sloping, benching or protective systems are found in ______________. 

A.  Safety Manuals

C.  CFR 1926.652 (OSHA Construction Standards)

B.  Tabulated data

D.  None of the above

(S) Means the answer can be plural or singular in nature
100.
Whenever support systems, _________________, or other protective systems are being used, a written copy of the manufacturer's specifications, recommendations, and limitations sheet shall be available at the job site.

A.  Shield systems

C.  Ramps

B.  Tabulated data

D.  None of the above

Excavation Protection Systems

101.
There are three basic protective systems for excavations and trenches. They are sloping and benching systems, _____________, and shields.

A.  Shoring


C.  Attendants

B.  Ramps


D.  None of the above

Inspections

102.
The excavations, adjacent areas, and protective systems shall be inspected daily by the _________________.

A.  Contractor

C.  Competent person

B.  Employees

D.  None of the above 

Water Quality Section

Three Types of Public Water Systems 

​103.  Provides water to the same population year-round for example: homes, apartment buildings.
A.  TMCWS
C.  NOT CWS 

B.  CWSs 
D.  None of the above

104.  Approximately 85,000 systems

A.  TOCWS
C.  NTNCWSs 

B.  CWSs 
D.  None of the above

Managing Water Quality at the Source

105.  Contingent upon the region, source water may have several restrictions of use as part of a Water Shed Management Plan. In some areas, it may be restricted from recreational use, discharge or runoff from agriculture, or________________________ . 
A.  Excess nutrients

C.  Industrial and wastewater discharge
B.  Biological actions 

D.  None of the above
Physical Characteristics of Water
106.  Physical characteristics are the elements found that are considered alkali, metals, and non-metals such as carbonates, fluoride,___________________. The consumer relates it to scaling of faucets or staining. 
A.  pH and alkalinity

C.  Powdered activated carbon and chlorine
B.  Sulfides or acids

D.  None of the above
Turbidity Introduction 

107.  One physical feature of water is turbidity. A measure of the cloudiness of water caused by___________________. The cloudy appearance of water caused by the presence of tiny particles. 
A.  Suspended particles
C.  Temperature fluctuation 

B.  Variations


D.  None of the above
(S) Means the answer can be plural or singular in nature
pH Testing Section

108.  When an atom loses ___________and thus has more protons than electrons, the atom is a positively-charged ion or cation. 

A.  A proton

C.  An electron 

B.  Charge

D.  None of the above

109.  Pure water has a pH very close to?

A.  7

C.  7.7

B. 7.5

D.  None of the above

110.  Since pH is a logarithmic scale, a difference of one pH unit is equivalent to _______________ difference in hydrogen ion concentration
A.  1
 C.  10

B. .1
 D.  None of the above
Objections to Hard Water

Scale Formation

111.  Hard water forms scale, usually__________________, which causes a variety of problems. Left to dry on the surface of glassware and plumbing fixtures, including showers doors, faucets, and sink tops; hard water leaves unsightly white scale known as water spots. 
A.  Magnesium carbonate
C.  Calcite


B.  Calcium carbonate
D.  None of the above

What are Disinfection Byproducts (DBPs)? 
112.  Which of the following forms when disinfectants used to treat drinking water react with naturally occurring materials in the water?

A.  Chloramines

C.  Disinfection byproducts (DBPs)
B.  Humic and fulvic acids
D.  None of the above
Disinfection Byproduct Research and Regulations Summary

113.  _________________is unquestionably the most important step in the treatment of water for drinking water supplies. 
A. DBP(s)


C.  Disinfection
B. Turbidity (particle)

D.  None of the above
Bacteriological Monitoring Section

Contaminants that may be present in sources of drinking water include:  

114.  Which of the following like salts and metals, which can be naturally occurring or result from urban stormwater runoff, industrial or domestic wastewater discharges, oil and gas production, mining or farming?

A.  Radioactive contaminants   
C. Inorganic contaminants

B.  Pesticides and herbicides   
D. Microbial contaminants

Background

115.  Coliform bacteria and chlorine residual are the only routine sampling and monitoring requirements for small ground water systems with chlorination. The coliform bacteriological sampling is governed by the Coliform Reduction amendment of the SDWA. 

A.  True
B.  False 
(S) Means the answer can be plural or singular in nature
TCR 

116.  The TCR recommends most of the Public Water Systems (PWS) to monitor their distribution system for bacteria according to the written sample sitting plan for that system. 
A.  True
B.  False 

117.  Coliform contamination may occur anywhere in the system, possibly due to problems such as; high-pressure conditions, line fluctuations, or wells, and therefore routine monitoring is required. 

A.  True
B.  False 

Routine Sampling Requirements 

118.  Total coliform samples must be collected by PWSs at sites which are representative of water quality throughout the distribution system according to a written sample sitting plan subject to state review and revision. 

A.  True
B.  False 

119.  For PWSs collecting more than one sample per month, collect total coliform samples at regular intervals throughout the month, except that ground water systems serving 4,900 or fewer people may collect all required samples on a single day if the samples are taken from different sites.

A.  True
B.  False 

120.  Each total coliform-positive (TC+) routine sample must be tested for the presence of heterotrophic bacteria. 

A.  True
B.  False 

121.  If any TC+ sample is also E. coli-positive (EC+), then the EC+ sample result must be reported to the state by the end of the month that the PWS is notified. 

A.  True
B.  False 

122.  If any routine sample is TC+, repeat samples are required. – PWSs on quarterly or annual monitoring must take a minimum of one additional routine samples (known as additional routine monitoring) the quarter following a TC+ routine or repeat sample. 

A.  True
B.  False 

123.  Reduced monitoring is general available for PWSs using only surface water and serving 1,000 or fewer persons that meet certain additional PWS criteria.
A.  True
B.  False 

Dangerous Waterborne Microbes 
124.  Which of the following is a parasite that enters lakes and rivers through sewage and animal waste? It causes cryptosporidiosis, a mild gastrointestinal disease. The disease can be severe or fatal for people with severely weakened immune systems. 

A.  Coliform Bacteria 
C.  Septic
B.  Cryptosporidium
D.  None of the above
125. Which of the following are not necessarily agents of disease, fecal coliform bacteria may indicate the presence of disease-carrying organisms, which live in the same environment as the fecal coliform bacteria?
A.  Fecal coliform bacteria
C.  Protozoa
B.  Cryptosporidium

D.  None of the above
126.  Which of the following is a parasite that enters lakes and rivers through sewage and animal waste. It causes gastrointestinal illness (e.g. diarrhea, vomiting, and cramps)?

A.  Coliform Bacteria 
C.  Protozoa

B.  Cryptosporidium
D.  None of the above

127.  Which of the following is a species of the rod-shaped bacterial genus Shigella? 

A.  Fecal coliform bacteria
C.  Shigella dysenteriae
B.  Cryptosporidium

D.  None of the above
Bacteriological Monitoring Introduction

128.  Which of the following are usually harmless, occur in high densities in their natural environment and are easily cultured in relatively simple bacteriological media? 

A.  Indicator bacteria 
C.  Viruses 

B.  Amoebas   
D.  None of the above
129.  Indicators in common use today for routine monitoring of drinking water include total coliforms, fecal coliforms, and?

A.  Cryptosporidium 
C.  Escherichia coli (E. coli)



B.  Protozoa   

D.  None of the above
130.  According to the text, the routine microbiological analysis of your water is for? 

A.  Contamination
C.  Coliform bacteria

B.  Colloids
   
D.  None of the above
The three (3) primary types of samples are:

131.  A PWS has a second Level 1 Assessment within a rolling 12-month period. 

A.  Trigger: Level 1 Assessment 
C.  All of the above

B.  Trigger: Level 2 Assessment 
D.  None of the above

132.  A PWS on state-approved annual monitoring has a Level 1 Assessment trigger in 2 consecutive years.

A.  Trigger: Level 1 Assessment 
C.  All of the above

B.  Trigger: Level 2 Assessment 
D.  None of the above

133.  A PWS collecting fewer than 40 samples per month has 2 or more TC+ routine/ repeat samples in the same month. 

A.  Trigger: Level 1 Assessment 
C.  All of the above

B.  Trigger: Level 2 Assessment 
D.  None of the above

134.  A PWS fails to take every required repeat sample after any single TC+ sample

A.  Trigger: Level 1 Assessment 
C.  All of the above

B.  Trigger: Level 2 Assessment 
D.  None of the above

Heterotrophic Plate Count (Spread Plate Method)

135.  Which of the following provides a technique to quantify the bacteriological activity of a sample?

A.  Colonies
C.  Heterotrophic Plate Count

B.  Agar
D.  None of the above
Total Coliforms

136.  This MCL is based on the presence of total coliforms, and compliance is on a daily or weekly basis, depending on your water system type and state rule. 

A.  True
B.  False
137.  For systems that collect fewer than ______________ samples per month, no more than one sample per month may be positive. In other words, the second positive result (repeat or routine) in a month or quarter results in a MCL violation. 

A.  40

C.  200

B.  100

D.  None of the above
The following are acute violations:

138.  Which determines a  violation of nitrate?

A.  Presence
C.  MCLG


B.  MCL
D.  None of the above
Revised Total Coliform Rule (RTCR) Summary

139.  EPA published the Revised Total Coliform Rule (RTCR) in the Federal Register (FR) on February 13, 2013 (78 FR 10269). It is the revision to the 1989 Total Coliform Rule (TCR). 
A.  True
B.  False

140.  The RTCR upholds the purpose of the 1989 TCR to protect public health by ensuring the duplicity of the drinking water distribution system and monitoring for the absence of microbial contamination.

A.  True
B.  False 

141.  The RTCR establishes criteria for systems to qualify for and stay on for special increased monitoring, which could reduce water system problems for better system operation.

A.  True
B.  False 

142.  The water provider shall develop and follow a sample-siting plan that designates the PWS's collection schedule. This includes location of___________.

A.  Routine and repeat water samples
C.  Microbial contamination
B.  Reduced monitoring


D.  Repeat water samples

143.  The water provider shall collect _______________________on a regular basis (monthly, quarterly, annually). Have samples tested for the presence of total coliforms by a state certified laboratory.

A.  Routine water samples
C.  Microbial contamination
B.  Reduced monitoring
D.  Repeat water samples

144.  PN is required for violations incurred. Within required timeframes, the PWS must use the required health effects language and notify the public if they did not comply with certain requirements of the RTCR. The type of ____________ depends on the severity of the violation.

A.  CCR(s)
C.  MCL violation

B.  PN

D.  TC+ routine or repeat sample

145.  The RTCR requires public water systems that are vulnerable to microbial contamination to identify and fix problems. 

A.  True
B.  False 

146.  The water provider shall collect repeat samples (at least 3) for each TC+ positive routine sample.

A.  True
B.  False 

147.  For PWSs on quarterly or annual routine sampling, collect additional routine samples (at least 3) in the month after a ______________________ .
A.  CCR(s)
C.  Total coliform positive samples

B.  PN

D.  TC+ routine or repeat sample

148.  PWSs incur violations if they do not comply with the requirements of the RTCR. The violation types are essentially the same as under the TCR with few changes. The biggest change is no acute or monthly MCL violation for ______________only.

A.  CCR(s)
C.  Total coliform positive samples

B.  PN

D.  TC+ routine or repeat sample

149.  Community water systems (CWSs) must use specific language in their CCRs when they must conduct an assessment or if they incur________________.

A.  CCR(s)
C.  An E. coli MCL violation

B.  PN

D.  TC+ routine or repeat sample

Disinfection Key
150.  The RTCR requires 99.99% or 4 log inactivation of _____________ .
A.  Enteric viruses
C.  Giardia lamblia cysts

B.  Crypto

D.  None of the above

151.  The RTCR requires 99% or 2 log inactivation of _________________ .
A.  Enteric viruses
C.  Giardia lamblia cysts

B.  Crypto

D.  None of the above

152.  The RTCR requires 99.9% or 3 log inactivation of ___________________. 
A.  Enteric viruses
C.  Giardia lamblia cysts

B.  Crypto

D.  None of the above

Chain of Custody Procedures 

153.  If both parties involved in the transfer must sign, date and note the time on the chain of custody record, this is known as? 

A.  TC Plan 


C.  Samples transfer possession

B.  Sample siting plan 
D.  None of the above
154.  The recipient will then attach the _______________showing the transfer dates and times to the custody sheets. If the samples are split and sent to more than one laboratory, prepare a separate chain of custody record for each sample.

A.  Shipping invoices

C.  Sample siting plan

B.  Chain of custody release
D.  None of the above
Disinfection Section 
Chlorine’s Appearance and Odor 

155.  Chlorine is a greenish-yellow gas it will condense to an amber liquid at approximately ______________F or at high pressures. 

A.  -29.2 degrees

C.  29 degrees

B.  – 100 degrees

D.  None of the above
156.  Prolonged exposures to chlorine gas may result in?

A.  Moisture, steam, and water
C.  Olfactory fatigue
B.  Odor thresholds 


D.  None of the above
Chlorine Gas

Pathophysiology

157.  As far as chlorine safety and respiratory protection, the intermediate ___________of chlorine accounts for its effect on the upper airway and the lower respiratory tract. 

A.  Effects of Hydrochloric acid
C.  Water solubility
B.  Vapor from Chlorine gas

D.  None of the above
158.  Respiratory exposure to ____________ may be prolonged because its moderate water solubility may not cause upper airway symptoms for several minutes.

A.  Hydrochloric acid

C. Plasma exudation

B.  Chlorine gas

D.  None of the above
159.  The odor threshold for chlorine gas is approximately? 

A.  0.3-0.5 parts per million (ppm)
C.  3-5 parts per million (ppm)

B.  3 parts per million (ppm)

D.  None of the above
Mechanism of Activity 

160.  Chlorine gas feeds out of the cylinder through a gas regulator.  The cylinders are on a scale that operators use to measure the amount used each day.  The chains are used to prevent the tanks from falling over. 

A.  True
B.  False
Early Response to Chlorine Gas 

161.  If you mix ammonia with chlorine gas, this compound reacts to form___________. 

A.  Chloramine gas

C.  Sulfuric gas
B.  Chlorine gas

D.  None of the above
Reactivity

162.  Cylinders of chlorine may burst when exposed to elevated temperatures. When there is Chlorine in solution, this forms?

A.  Hydrogen sulfide

C.  A corrosive material

B.  Oxomonosilane

D.  None of the above
163.  What is formed when chlorine is in contact with combustible substances (such as gasoline and petroleum products, hydrocarbons, turpentine, alcohols, acetylene, hydrogen, ammonia, and sulfur), reducing agents, and finely divided metals? 

A.  Fires and explosions
C.  Moisture, steam, and water
B.  Odor thresholds 

D.  None of the above
164.  Contact between chlorine and arsenic, bismuth, boron, calcium, activated carbon, carbon disulfide, glycerol, hydrazine, iodine, methane, oxomonosilane, potassium
, propylene, and silicon should be avoided. 

A.  True  
B.  False

165.  Chlorine reacts with hydrogen sulfide and water to form this substance? 

A.  Hydrogen sulfide

C.  Chlorinates 

B.  Hydrochloric acid

D.  None of the above
166.  According to the text, chlorine is also incompatible with? 

A.  Plastic

C.  Moisture, steam, and water 
B.  Palladium

D.  None of the above
Flammability

167.  When there is a fire that involves Chlorine, the firefight should be fought downwind from the minimum distance possible. 

A.  True  
B.  False

168.  Keep unnecessary people away; isolate the hazard area and deny entry. For a massive fire in a cargo area, use unmanned hose holders or monitor nozzles; if this is impossible, withdraw from the area and let the fire burn. Emergency personnel should stay out of low areas and Ventilate closed spaces before entering. 

A.  True  
B.  False

169.  The effectiveness of chlorination depends on the _______________ of the water, the concentration of the chlorine solution added, the time that chlorine is in contact with the organism, and water quality. 

A.  Chlorine residual

C.  Oxygen

B.  Chlorine demand

D.  None of the above
170.  Chlorine may not be available for disinfection because _____________ in the water (like iron, manganese, hydrogen sulfide, and ammonia). 

A.  pH increases 




C.  Required contact time

B.  Part of it combines with other chemicals

D.  None of the above
171.  The amount of chlorine required to achieve disinfection and that reacts with the other chemicals is the? 

A.  Chlorine residual

C.  Free chlorine residual

B.  Chlorine demand

D.  None of the above
172.  Which term is used when disinfection decreases, as the concentration of the chlorine increases? 

A.  pH increases 



C.  Required contact time

B.  Chlorine level and water quality

D.  None of the above
173.  Chlorination is more effective as? 

A.  Water temperature increases
C. Water cools down

B.  Chlorine demand


D. None of the above
174.  Chlorination becomes more alkaline and is less effective as the? 

A.  Water's pH increases

C.  Required contact time is maximized

B.  Water quality increases

D.  None of the above
175.  Chlorination is less effective in?

A.  Clear water


C.  Day time

B.  Cloudy (turbid) water

D.  None of the above
176.  By adding a little more chlorine to what is already sufficient, this action will generally result in ____________that can be measured easily. 

A.  pH increases 


C.  Required contact time

B.  A free chlorine residual

D.  None of the above
Chlorination Chemistry

177.  The hypochlorite ion is a much weaker disinfecting agent than Hypochlorous acid, about 100 times less effective.

A.  True  
B.  False

178.  According to the text, pH and temperature affect the ratio of hypochlorous acid to hypochlorite ions. As the temperature is decreased, the ___________________increases. 

A.  Reduction Ratio


C. "CT" disinfection concept
B.  Ratio of hypochlorous acid
D.  None of the above
179.  Under normal water conditions, hypochlorous acid will also chemically react and break down into the hypochlorite ion. 

A.  True  
B.  False

180.  Although the ratio of _____________ is greater at lower temperatures, pathogenic organisms are actually harder to kill. 

A.  Hypochlorous acid

C. Total chlorine
B.  The amount of chlorine 
D.  None of the above
181.  If all other things were equal, _________ and a lower pH are more conducive to chlorine disinfection.

A.  Lower pH


C.  Higher water temperatures

B.  Hypochlorous acid

D.  None of the above
182.  All three forms of chlorine produce Sodium hypochlorite when added to water. 

A.  True  
B.  False

183.  Hypochlorous acid is a strong acid but a weak disinfecting agent. The amount of hypochlorous acid depends on the pH and temperature of the water. 

A.  True  
B.  False

Chlorine DDBP 
184.  These term means that chlorine is present as ClINCLUDEPICTURE "F:\\..\\..\\..\\Operator cert\\PDH\\wrap40.xbm" \* MERGEFORMAT \d \z", HOCl, and OCl¯ is called ______________, and that which is bound but still effective is ________________. 

A. Free available chlorine and Total


B. Free and Residual





C. Free available chlorine and Combined Chlorine
D.  None of the above
185.  Chloramines are formed by reactions with?

A.  Acid and Cl2

C.  Folic Acid and Cl2

B.  Ammonia and Cl2

D.  None of the above


Types of Residual

186. Which of the following is all chlorine that is available for disinfection?

A. Chlorine residual
C.  Total chlorine

B. Chlorine demand
D.  None of the above
Chlorine Exposure Limits

187.  What is OSHA’s PEL?  

A.  10 PPM

C.  1,000 PPM

B.  1 PPM

D.  None of the above

188.  Chlorine’s Physical and chemical properties: A yellowish green, nonflammable and liquefied gas with an unpleasant and irritating smell.  

A.  True
B.  False

189.  Liquid chlorine is about __________ times heavier than water 

A.  1.5

C.  2.5

B.  10 

D.  None of the above

190.  Gaseous chlorine is about _____________ times heavier than air.
A.  1.5

C.  2.5

B.  10 

D.  None of the above

Alternate Disinfectants - Chloramine

191.  It is recommended that Chloramine be used in conjunction with a stronger disinfectant. It is best utilized as a?
A. Chloramine 

C.  Stable distribution system disinfectant 

B.  T10 value disinfectant
D.  None of the above
192.  In the production of ____________________, the ammonia residuals in the finished water, when fed in excess of stoichiometric amount needed, should be limited to inhibit growth of nitrifying bacteria.

A.  Dry sodium chlorite 
C.  Ammonia residual(s) 

B.  Chloramines

D.  None of the above 

Chlorine Dioxide

193.  Which term provides good Giardia and virus protection but its use is limited by the restriction on the maximum residual of 0.5 mg/L ClO2/chlorite/chlorate allowed in finished water?

A.  Chlorinated byproducts
C.  Ammonia residual(s) 

B.  Chlorine dioxide 

D.  None of the above 

194.  If chlorine dioxide is being used as an oxidant, the preferred method of generation is to entrain or ____________________ into a packed reaction chamber with a 25% aqueous solution of sodium chlorite (NaClO2).

A.  Chloramine 

C.  Chlorine dioxide 

B.  Chlorine gas

D.  None of the above
195.  Which chemical is explosive and can cause fires in feed equipment if leaking solutions or spills are allowed to dry out?

A.  Dry sodium chlorite 
C.  Ammonia 

B.  Chlorine dioxide 

D.  None of the above 

196.  Chlorine dioxide may be used for either taste or odor control or as a?

A.  Chloramine 
D.  Gas  

B.  Pre-disinfectant
D.  None of the above
197.  Total residual oxidants (including chlorine dioxide and chlorite but excluding Chlorine dioxide) shall not exceed 0.50 mg/L during normal operation or 0.30 mg/L (including chlorine dioxide, chlorite and chlorate) during periods of extreme variations in the raw water supply.

A.  True   
B.  False

Ozone

198.  When determining Ozone CT (contact time) values must be determined for the ozone basin alone; an accurate _____________ must be obtained for the contact chamber, and residual levels. 

A.  Residual 
C.  Contact time 

B.  T10 value
D.  None of the above
199.  Ozone does not provide a system residual and should be used as a primary disinfectant only in conjunction with? 

A.  Dry sodium chlorite 
C.  Free and/or combined chlorine

B.  Chlorine dioxide 

D.  None of the above 

200.  Ozone does not produce chlorinated byproducts (such as trihalomethanes) but it may cause an increase in such byproduct formation if it is fed ahead of free chlorine; ozone may also produce its own oxygenated byproducts such as Cl2 + NH4. 

A.  True   
B.  False
When Finished with Your Assignment

REQUIRED DOCUMENTS

Please scan the Registration Page, Answer Key, Survey and Driver’s License and email it to info@TLCH2O.com.

IPhone Scanning Instructions

If you are unable to scan, take a photo of these documents with your iPhone and send these photos to TLC, info@TLCH2O.com.

FAX

If you are unable to scan and email, please fax these to TLC, if you fax, call to confirm that we received your paperwork. (928) 468-0675

34
8
Distribution 8 Hours Assignment                                                      TLC © 1/7/2026    www.abctlc.com


